MC68HC11Floating Point PackageWritten ByGordon Doughman� 1986 Gordon Doughman1.0 Introduction:	The MC68HC11 is a very powerful and capable single chip microcomputer.  Its concise instruction setcombined with 6 powerful addressing modes, true bit manipulation, 16-bit arithmetic operations and a second 16-bitindex register make it ideal for control applications requiring  both high speed I/O and high speed calculations.	While most applications can be implemented by using the 16-bit integer precision of the MC68HC11,certain applications or algorithms may be difficult or impossible to implement without floating point math.  The goal inwriting the MC68HC11 floating point package was to provide a fast, flexible way to do floating point math for just suchapplications.	The HC11 floating point package (HC11FP) implements more than just the four basic math functions (add,subtract, multiply, and divide) it also provides routines to convert from ASCII to floating point and from floating pointto ASCII.  For those applications that require it, the three basic trig functions SINe, COSine, and TANgent areprovided along with some trig utility functions for converting to and from both radians and degrees.  The square rootfunction is also included.	For those applications that can benefit by using both integer and floating point operations, there areroutines to convert to and from integer and floating point format.	The entire floating point package requires just a little over 2k bytes of memory and only requires tenbytes of page zero RAM in addition to stack RAM.  All temporary variables needed by the floating point routines,reside on the stack.  This feature makes the routines completely reentrant as long as the ten bytes of page zero RAMare saved before using any of the routines.  This will allow both interrupt routines and main line programs to use thefloating point package with out interfering with one another.2.0 Floating Point Format:2.1 Floating Point Accumulator Format:	The ten bytes of page zero RAM are used for two software floating point accumulators FPACC1 andFPACC2.  Each 5 byte accumulator consists of a one byte exponent, a three byte mantissa, and one byte that is usedto indicate the mantissa sign.	The exponent byte is used to indicate the position of the binary point  and is biased by decimal 128 ($80)to make floating point comparisons easier. This one byte exponent gives a dynamic range of about 1 � 10�38.	The mantissa consists of three bytes (24 bits) and is used to hold both the integer and fractional portionof the floating point number.  The mantissa is always assumed to be "normalized" (i.e. most significant bit of the mostsignificant byte a 1).  A twenty-four bit mantissa will provide slightly more than seven decimal digits of precision.	A separate byte is used to indicate the sign of the mantissa rather than keeping it in two's complementform so that unsigned arithmetic operations may be used when manipulating the mantissa.  A positive mantissa isindicated by this byte being equal to zero ($00).  A negative mantissa is indicated by this byte being equal to -1 ($FF).	FPACC1	82 C90FDB 00	+3.1415927	FPACC2	82 C90FDB FF	-3.14159272.2 "Memory Format":	The way that floating point numbers are stored in memory or the "memory format" of a floating pointnumber is slightly different than its floating point accumulator format.  In order to save memory, floating point numbersare stored in memory in a format called "hidden bit normalized form".	In this format, the number is stored into four consecutive bytes with the exponent residing at the lowestaddress.  The mantissa is stored in the next three consecutive bytes with the most significant byte stored in thelowest address.  Since the most significant bit of the mantissa in a normalized floating point number is always "1", thisbit can be used to store the sign of the mantissa.  This results in positive numbers having the most significant bit ofthe mantissa cleared (0) and negative numbers having their most significant bit set (1).  An example follows:		82 490FDB	+3.1415927		82 C90FDB	-3.1415927	There are four routines that can be used to save and load the floating point accumulators and at the sametime convert between the floating point accumulator and memory format.  These routines are discussed in detail inanother section.3.0 Errors:	There are 7 error conditions that may be returned by the HC11 floating point package.  When an erroroccurs, the condition is indicated to the calling program by setting the carry bit in the condition code register andreturning an error code in the A-accumulator.  The error codes and their meanings are explained below.	Error #	Meaning	1	Format error in ASCII to floating point conversion.	2	Floating point overflow.	3	Floating point underflow.	4	Division by zero (0).	5	Floating point number too large or small to convert		to integer.	6	Square root of a negative number.	7	TAN of �/2 (90�)Note: None of the routines check for valid floating point numbers in either FPACC1 or FPACC2.  Having illegalfloating point values in the floating point accumulators will produce unpredictable results.4.0 Floating Point Routines:	The following sections provides a description of each routine in the floating point package.  Theinformation provided includes the subroutine name, operation performed, subroutine size, stack space required,other subroutines that are called, input, output, and possible error conditions.	The Stack Space required by the subroutine includes not only that required for the particular routineslocal variables, but also stack space that is used by any other subroutines that are called including return addresses. Note that the trig functions require a good deal of stack space.	Since some applications may not require all the routines provided in the floating point package, thedescription of each routine includes a section giving the names of other subroutines that it calls.  This makes it easyto determine exactly which subroutines are required for a particular function.4.1 ASCII-to-Floating Point Conversion:Subroutine Name:	ASCFLTOperation:	ASCII (X) � FPACC1Size:	352 Bytes (includes NUMERIC subroutine)Stack Space:	14 BytesCalls:	NUMERIC, FPNORM, FLTMUL, PSHFPAC2, PULFPAC2Input:	X-register points to ASCII string to convert.Output:	FPACC1 contains the floating point number.Error conditions:	Floating point format error may be returned.Notes: 	This routine converts an ASCII floating point number to the format required by all ofthe floating point routines.  Conversion stops either when a non-decimal character isencountered before the exponent or after one or two exponent digits have beenconverted.  The input format is very flexible.  Some examples are shown below.		20.095		0.125		7.2984E+10		167.824E5		005.9357E-7		5004.2 Floating Point Multiply:Subroutine Name:	FLTMULOperation:	FPACC1 � FPACC2 � FPACC1Size:	169 BytesStack Space:	10 BytesCalls:	PSHFPAC2, PULFPAC2, CHCK0Input:	FPACC1 and FPACC2 contain the numbers to be multiplied.Output:	FPACC1 contains the product of the two floating point accumulators.  FPACC2remains unchanged.Error conditions:	Overflow, Underflow.4.3 Floating Point Add:Subroutine Name:	FLTADDOperation:	FPACC1 + FPACC2 � FPACC1Size:	194 BytesStack Space:	6 BytesCalls:	PSHFPAC2, PULFPAC2, CHCK0Input:	FPACC1 and FPACC2 contain the numbers to be added.Output:	FPACC1 contains the sum of the two numbers.  FPACC2 remains unchanged.Error conditions:	Overflow, Underflow.Notes:	The floating point add routine performs full signed addition.  Both floating pointaccumulators may have mantissas  with the same or different sign.4.4 Floating Point Subtract:Subroutine Name:	FLTSUBOperation:	FPACC1 - FPACC2 � FPACC1Size:	12 BytesStack Space:	8 BytesCalls:	FLTADDInput:	FPACC1 and FPACC2 contain the numbers to be subtracted.Output:	FPACC1 contains the difference of the two numbers (FPACC1-FPACC2).  FPACC2remains unchanged.Error conditions:	Overflow, Underflow.Notes:	Since FLTADD performs full signed addition, the floating point subtract routineinverts the sign byte of FPACC2, calls FLTADD and then changes the sign ofFPACC2 back to what it was originally.4.5 Floating Point Divide:Subroutine Name:	FLTDIVOperation:	FPACC1 � FPACC2 � FPACC1Size:	209 BytesStack Space:	11 BytesCalls:	PSHFPAC2, PULFPAC2Input:	FPACC1 and FPACC2 contain the divisor and dividend respectively.Output:	FPACC1 contains the quotient. FPACC2 remains unchanged.Error Conditions:	Divide by zero, Overflow, Underflow4.6 Floating Point to ASCII Conversion:Subroutine Name:	FLTASCOperation:	FPACC1 � (X)Size:	370 BytesStack Space:	28 BytesCalls:	FLTMUL, FLTCMP, PSHFPAC2, PULFPAC2Input:	FPACC1 contains the number to be converted to an ASCII string. The index registerX points to a 14 byte string buffer.Output:	The buffer pointed to by the X index register contains an ASCII string that representsthe number in FPACC1.  The string is terminated with a zero (0) byte and the Xregister points to the start of the string.Error Conditions:	None.4.7 Floating Point Compare:Subroutine Name:	FLTCMPOperation:	FPACC1 - FPACC2Size:	42 BytesStack Space:	NoneCalls:	NoneInput:	FPACC1 and FPACC2 contain the numbers to be compared.Output:	Condition codes are properly set so that all branch instructions may be used to alterprogram flow.  FPACC1 and FPACC2 remain unchanged.Error Conditions:	None.4.8 Unsigned Integer to Floating Point:Subroutine Name:	UINT2FLTOperation:	(16-bit unsigned integer) � FPACC1Size:	18 BytesStack Space:	6 BytesCalls:	FPNORM,

CHCK0Input:	The lower 16-bits of the FPACC1 mantissa contain an unsigned 16-bit integer.Output:	FPACC1 contains the floating point representation of the 16-bit unsigned integer.Error Conditions:	None.4.9 Signed Integer to Floating Point:Subroutine Name:	SINT2FLTOperation:	(16-bit signed integer) � FPACC1Size:	24 BytesStack Space:	7 BytesCalls:	UINT2FLTInput:	The lower 16-bits of the FPACC1 mantissa contain a signed 16-bit integer.Output:	FPACC1 contains the floating point representation of the 16-bit signed integer.Error Conditions:	None.4.10 Floating Point to Integer:Subroutine Name:	FLT2INTOperation:	FPACC1 � (16-bit signed or unsigned integer)Size:	74 BytesStack Space:	2 BytesCalls:	CHCK0Input:	FPACC1 may contain a floating point number in the range 65535 � FPACC1 � -32767.Output:	The lower 16-bits of the FPACC1 mantissa will contain a 16-bit signed or unsignednumber.Error Conditions:	None.Notes:	If the floating point number  in FPACC1 is positive, it will be converted to an unsignedinteger.  If the number is negative it will be converted to a signed twos complementinteger. This type of conversion will allow 16-bit addresses to be represented aspositive numbers in floating point format.  Any fractional part of the floating pointnumber is discarded.4.11 Transfer FPACC1 to FPACC2:Subroutine Name:	TFR1TO2Operation:	FPACC1 � FPACC2Size:	13 BytesStack Space:	0 BytesCalls:	NoneInput:	FPACC1 contains a floating point number.Output:	FPACC2 contains the same number as FPACC1.Error Conditions:	None.5.0 Floating Point Functions:	The following section describes the supplied floating point functions, returned results, and possible errorconditions.  Note that even though the Taylor series which is used to calculate the trig functions requires that theinput angle be expressed in radians, less precision is lost through angle reduction if the angle being reduced isexpressed in degrees. Once the angle is reduced, the DEG2RAD subroutine is called to convert the angle to radians.	To reduce the number of factors in the Taylor expansion series all angles are reduced to fall between 0�and 45� by the ANGRED subroutine. This subroutine returns the reduced angle in FPACC1 along with the quad numberthat the original angle was in and a flag that tells the calling routine whether it actually needs to calculate the sine orthe cosine of the reduced angle to obtain the proper answer.5.1 Square Root:Subroutine Name:	FLTSQROperation:	�FPACC1 � FPACC1Size:	104 BytesStack Space:	21 BytesCalls:	TFR1TO2, FLTDIV, FLTADD, PSHFPAC2, PULFPAC2Input:	FPACC1 contains a valid floating point number.Output:	FPACC1 contains the square root of the original number.  FPACC2 is unchanged.Error Conditions:	NSQRTERR is returned if the number in FPACC1 is negative and FPACC1 remainsunchanged.5.2 Sine:Subroutine Name:	FLTSINOperation:	SIN(FPACC1) � FPACC1Size:	380 Bytes ( Includes SINCOS subroutine )Stack Space:	50 BytesCalls:	ANGRED, SINCOS, DEG2RAD, PSHFPAC2, PULFPAC2Input:	FPACC1 contains an angle in radians in the range -2� � FPACC1 � +2�.Output:	FPACC1 contains the sine of FPACC1 and FPACC2 remains unchanged.Error Conditions:	None.Notes:	The Taylor Expansion Series is used to calculate the sine of the angle between 0 and45� (��4).  The subroutine ANGRED is called to reduce the input angle to within thisrange.  Spot checks show a maximum error of +1.5�10-7 throughout the input range.5.3 Cosine:Subroutine Name:	FLTCOSOperation:	COS(FPACC1) � FPACC1Size:	384 Bytes ( Includes SINCOS subroutine )Stack Space:	50 BytesCalls:	ANGRED, FLTSIN, DEG2RAD, PSHFPAC2Input:	FPACC1 contains an angle in radians in the range -2� � FPACC1 � +2�.Output:	FPACC1 contains the cosine of FPACC1 and FPACC2 remains unchanged.Error Conditions:	None.Notes:	The Taylor Expansion Series is used to calculate the cosine of the angle between 0and 45� (��4).  The subroutine ANGRED is called to reduce the input angle to withinthis range.  Spot checks show a maximum error of +1.5�10-7 throughout the inputrange.5.4 Tangent:Subroutine Name:	FLTTANOperation:	TAN(FPACC1) � FPACC1Size:	35 Bytes ( Also requires FLTSIN and FLTCOS )Stack Space:	56 BytesCalls:	TFR1TO2, EXG1AND2, FLTSIN, FLTCOS, FLTDIV, PSHFPAC2, PULFPAC2Input:	FPACC1 contains an angle in radians in the range -2� � FPACC1 � +2�.Output:	FPACC1 contains the tangent of the input angle and FPACC2 remains unchanged.Error Conditions:	Returns largest legal number if tangent of ���2 is attempted.Notes:	The tangent of the input angle is calculated by first obtaining the sine and cosine ofthe input angle and  then using the following formula: TAN=SIN�COS.  At 89.9� thetangent function is only accurate to 5 decimal digits.  For angles greater than 89.9�accuracy decreases rapidly.5.5 Degrees to Radians Conversion:Subroutine Name:	DEG2RADOperation:	FPACC1 � � � 180 � FPACC1Size:	15 BytesStack Space:	16 BytesCalls:	GETFPAC2,FLTMULInput:	Any valid floating point number representing an angle in degrees.Output:	Input angles equivalent in radians.Error Conditions:	None.5.6 Radians to Degrees Conversion:Subroutine Name:	RAD2DEGOperation:	FPACC1 � 180 � � � FPACC1Size:	8 Bytes ( Also requires DEG2RAD subroutine )Stack Space:	16 BytesCalls:	DEG2RADInput:	Any valid floating point number representing an angle in radians.Output:	Input angles equivalent in degrees.Error Conditions:	Overflow, Underflow.5.7 PI:Subroutine Name:	GETPIOperation:	� � FPACC1Size:	6 BytesStack space:	NoneInput:	NoneOutput:	The value of � is returned in FPACC1.Error Conditions:	NoneNotes:	This routine should be used to obtain the value of � if it is required in calculationssince it is accurate to the full 24 bits of the mantissa.6.0 Format Conversion Routines:	As discussed in section 2.1 and 2.2 the format for floating point numbers as they appear in the floatingpoint accumulators is different than the way numbers are stored in memory.  This was done primarily to save memorywhen a large number of floating point variables are used in a program.  Four routines are provided to convert to andfrom the different formats while at the same time moving a number into or out of the floating point accumulators.  Byalways using these routines to move numbers into and out of the floating point accumulators it would be extremelyeasy to adapt this floating point package to work with any other floating point format.	One example might be to interface this package with code produced by Motorola's 68HC11 'C' compiler. The Motorola 'C' compiler generates code for single precision floating point numbers whose internal format is thatdefined by the IEEE Standard for Binary Floating Point Arithmetic.  By rewriting the four routines described below theIEEE format could be easily converted to the format required by this floating point package.6.1 Get FPACC(x):Subroutine Name:	GETFPAC1 & GETFPAC2Operation:	(X) � FPACC1; (X) � FPACC2Size:	22 Bytes each.Stack Space:	NoneInput:	The X index register points to the 'memory formated' number to be moved into thefloating point accumulator.Output:	The number pointed to by X is in the specifyed floating point accumulator.Error Conditions:	None6.1 Put FPACC(x):Subroutine Name:	PUTFPAC1 & PUTFPAC2Operation:	FPACC1 � (X); FPACC2 � (X)Size:	22 Bytes each.Stack Space:	NoneInput:	The X index register points to four consecutive memory locations where the numberwill be stored.Output:	The floating point accumulator is moved into consecutive memory locations pointedto by the X index register.Error Conditions:	None######################################################