/*! \page porting Porting GEL

Porting should be relatively easy.  There are three points to

take care of:

<ul>

<li>\ref portIncludes

<li>\ref portFlags

<li>\ref portMemory

</ul>

*/

/*! \page portIncludes Board specific includes

GEL includes are organized so that it is possible to define board

specific information (#defines) and functions.  The board specific

includes must be located in:

<pre>

include/asm-m68hc11/arch-<board>

</pre>

or

<pre>

include/asm-m68hc12/arch-<board>

</pre>

The board specific includes are searched first when compiling GEL.

This means that, in theory, it is possible to override any GEL include

(although this is not recommended).  The board specific includes

should contain the following:

<TABLE BORDER="0">

<TR><TD WIDTH="30%" VALIGN="TOP">

arch/param.h

</TD><TD>

This file is included from <sys/param.h>.

You can override all #define specified in

either <asm-m68hc11/param.h> or <asm-m68hc12.h>.

</TD></TR>

<TR><TD WIDTH="30%" VALIGN="TOP">

arch/interrupts.h

</TD><TD>

This is included from <sys/interrupts.h>.

You must implement the `set_interrupt_handler'

function (as an inline).

</TD></TR>

<TR><TD WIDTH="30%" VALIGN="TOP">

arch/exit.h

</TD><TD>

This is included from <sys/exit.h>.

You must implement the `_exit'

function (as an inline).

</TD></TR>

</TABLE>

*/

/*! \page portFlags Board Specific Compilation Flags

Board specific compilation flags must be specified in the file:

<pre>

config/<cpu>-<board>/make.defs

</pre>

In general, it is recommended to specify additional flags and

avoid overriding completely existing flags.  In other words,

the recommended way is to use += in macro definition.  For example:

<pre>

CFLAGS+=-fomit-frame-pointer

</pre>

The board specific compilation flags must declare at least the

following make variables:

<TABLE BORDER="0">

<TR><TD WIDTH="30%" VALIGN="TOP">

CPU

</TD><TD>

This is the name of the CPU architecture.

Choose either m68hc11 or m68hc12.

</TD></TR>

<TR><TD WIDTH="30%" VALIGN="TOP">

BOARD

</TD><TD>

This is the board name extension.

The name is important and is used to construct

the compiler include directive to access to

the board specific includes.

</TD></TR>

</TABLE>

*/

/*! \page portMemory Board Memory Banks definitions

The board memory banks definitions tells the compiler what is the

address of rom and ram.  The linker uses this information to know

where the program code and program data must be put in memory (ROM or RAM).

The definitions is made in a file named memory.x.  The memory.x file

contains in general something like:

<pre>

MEMORY

{

page0 (rwx) : ORIGIN = 0x0, LENGTH = 256

text  (rx)  : ORIGIN = 0x8000, LENGTH = 0x8000

data        : ORIGIN = 0x1040, LENGTH = 0x8000-0x1040

}

PROVIDE (_stack = 0x8000 - 1);

</pre>

The \b page0 definition should not be changed.

The \b text definition represents the memory area where the program code

can be put.

The \b data definition represents the memory area for program data

(initialized and uninitialized data).

The easiest way to adapt the examples for your board is probably

to copy the m68hc11-32k directory to another directory and update

the memory.x and optionally the make.defs file to define the

characteristics of your board.

*/