T194 Encryption Table of WinDaqStarter Kit DI-194

All the info provided here is "AS IS", please do not email DATAQ for further support/explanation

If you have questions regarding programming this device, click here.
(Samples in various programming languages are available)

|  | B7 | B6 | B5 | B4 | B3 | B2 | B1 | B0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte1 | A0 | 0 | 0 | 0 | D2 | D1 | D0 | 0 |
| Byte2 | A7 | A6 | A5 | A4 | A3 | A2 | A1 | 1 |
| Byte3 | B0 | 0 | 0 | 0 | D2 | D1 | D0 | 1 |
| Byte4 | B7 | B6 | B5 | B4 | B3 | B2 | B1 | 1 |
| Byte5 | C0 | 0 | 0 | 0 | D2 | D1 | D0 | 1 |
| Byte6 | C7 | C6 | C5 | C4 | C3 | C2 | C1 | 1 |
| Byte7 | E0 | 0 | 0 | 0 | D2 | D1 | D0 | 1 |
| Byte8 | E7 | E6 | E5 | E4 | E3 | E2 | E1 | 1 |
| Note | Ax is the xth bit of ADC reading from channel one Bx is the xth bit of ADC reading from channel two if enabled Cx is the xth bit of ADC reading from channel three if enabled Ex is the xth bit of ADC reading from channel four if enabled Dx is the digital input bits To program DI-194 under VisualBASIC or LabView, please click here |  |  |  |  |  |  |  |
| Commands | Cmd; Data; Default; Action. C; 0 thru 15; 0; Channels 000 thru 1111. D; 0, 1; 0; 0 = output square wave 1= input. L; 0...255; 0; Ls byte of counter. M; 0...255; 0; Ms byte of counter. S; 0, 1; 0; ADC stop, start. R; Z; reset. E; key; N.A.; If you don't have the key when you got the unit, you can purchase it from DATAQ for $15.. N; Z; Return 10 bytes serial number of the device. If you run WinDaq, you can also inqure the serial number from the About dialog box. Send NULL before any command, which can be sent out by keying in <Ctrl-2> under VT100 mode if you are using HyperTerminal. |