Lord Tactile Sensor Driver Program

For LTS 200I Tactile Sensor With Rev 1.162 Software

By: Dan Kohn and Alf Bowser

For: Clark Pick (Instructor Sir Sandford Flemming College

Dec. 5,1986

This  program  demonstrates  the uses of a Tactile  Sensor.  To  run  the

program   make   sure   the  sensor  is  on and that  the  pad  is  clear  for

installation  of  the sensor.  After the menu comes on you can select on of  6

options. They are:

1) Get data from the sensor and save it on disk.

This   allows  you to get data from the sensor and  save  it   for

later use.  It  will also show you a view of the part with a color

scale  representing  the amount of force applied to the sensor  at

each   point.   Make  sure you have a part on  the  sensor  before

running this command.

2) Show 3 dimensional view of data on disk.

For this command you must have run command 1 first. After you have

a  part saved to disk this command will draw a 3 dimensional  view

of the surface of the part that is touching the sensor pad.

3) Compare part on disk to new part.

For this command you must have run command 1 first. After you have

save  a part to disk this command will compare the new part to the

one on the disk.  Once the command is picked it will ask you for a

tolerance  (between 0 and 100).  Make sure you have a part on  the

pad  before hitting return.  After you run this command  the  same

screen that appeared on command 1 will appear again, but after the

scan  is completed information will appear at the right hand  side

of  the screen informing you of the area of the old part,  area of

the  new  part  and weather or not the  part  passed  (within  the

tolerance you picked).

Note:   The   total  area of your part makes a difference  on  the

outcome.  If  your good part covers the entire sensor pad and  the

next part is missing one pixel and you use a  10%  tolerance,  the

part will be accepted because 160 +/- 10% gives a range  of 144 to

176,   and 159 (full sensor-1) is within that range.  But  on  the

other  hand  if you have a good part with 10 pixels  on  the   pad

covered  and a tolerance of 10% the range of the  part   it   will

except   is  9  to 11 pixels.   Therefor this algorithm  is   more

accurate for full scan parts.

4) Edge detection.

Before  running this command make sure the part you want to use is

already on the pad. Once the command is entered the screen that is

used  in command 1 is shown.  After the sensor sends all the  data

the computer will show and angle of rotation and an offset to  the

center  of the part that a robot would have to move to  place  the

part  in the desired location. This program assumes you would want

the part in the exact center of the pad.

Notes:  The  part  must cover the pad from one side to the  other.

Also the part must be placed on the pad from row zero up for  this

algorithm  to  work.  Only one direction and the angle  was  given

because the other direction would be a part dependent calculation.

5) Help.

Shows this information.

6) End program.

Stops execution of the program.

General notes:

The  sensor  must be on when the program is executed and must  remain  on

until  you  are done with the program (you cannot turn the sensor on  and

off during program operation)

You  can use the print screen key to get a printout of the 3  dimensional

view.