.op

.mt 0

.pl 57

Lord Tactile Sensor Driver Program

For LTS 200I Tactile Sensor With Rev 1.162 Software

By: Dan Kohn and Alf Bowser

For: Clark Pick

Dec. 5,1986

Table Of Contents

Running the program............................... 1

Commands and their use............................ 1

General notes..................................... 2

Program notes..................................... 3

Modifying the program............................. 5

Program Listing.......................... Appendix A

Autoexec file............................ Appendix B

Power supply and cable................... Appendix C

Running the Lord Sensor Demo Program

This   program  demonstrates  the uses of a Tactile  Sensor.

To  run  the program   make   sure   the  sensor is connected  to

the  power  supply and computer properly (refer to appendix C  of

this  report  for power supply and cable information)  Also  make

sure  that the sensor pad  is  clear  for installation.   To  run

the program put the disk marked LORD SENSOR DEMO PROGRAM into the

A: drive of an IBM XT with and RGB monitor and an installed Com1:

port.   Then  hold  down  the  Ctrl-Alt-Del  keys  simultaneously

(reboot). This will automatically remind you to turn the power to

the  sensor  on and,  after you hit any key,  will  load  in  the

program.  The first screen that comes on will tell you that it is

initializing the sensor. After initialization is complete it will

ask  you to hit any key.  At this time if you want to get further

instructions  type  the letter I.  This will give you a  list  of

commands and how to use them.  If you hit any other key, or after

you are done reading the help screens, the Menu will appear. From

this menu you can access 6 commands. They are:

1) Get data from the sensor and save it on disk.

This    allows  you to get data from the  sensor  and

save   it   for later use.   It  will also show you a

view  of  the part with a color  scale   representing

the  amount of force applied to the sensor   at  each

point.    Make   sure you have a part on  the  sensor

before running this command.

2) Show 3 dimensional view of data on disk.

For  this command you must have run command 1  first.

After  you  have a  part saved to disk  this  command

will draw a 3 dimensional  view of the surface of the

part that is touching the sensor pad.

3) Compare part on disk to new part.

For  this command you must have run command 1  first.

After you have save  a part to disk this command will

compare  the new part to the one on the  disk.   Once

the command is picked it will ask you for a tolerance

(between  0 and 100).   Make sure you have a part  on

the  pad  before hitting return.   After you run this

command  the  same screen that appeared on command  1

will  appear again,  but after the scan  is completed

information  will appear at the right hand   side  of

the screen informing you of the area of the old part,

area  of the  new  part  and weather or not the  part

passed  (within  the tolerance you picked).

Note:    The    total   area  of your  part  makes  a

difference   on   the outcome.   If  your  good  part

covers  the entire sensor pad and  the next  part  is

missing one pixel and you use a  10%  tolerance,  the

part  will  be accepted because 160 +/- 10%  gives  a

range   of 144 to 176,    and 159 (full sensor-1)  is

within that range.   But  on  the other  hand  if you

have  a  good  part  with 10 pixels   on   the    pad

covered   and  a tolerance of 10% the  range  of  the

part    it    will   except   is  9   to  11  pixels.

Therefor  this algorithm  is more  accurate for  full

scan parts.

4) Edge detection.

Before   running this command make sure the part  you

want  to use is already on the pad.  Once the command

is entered the screen that is  used  in command 1  is

shown.   After  the  sensor sends all the   data  the

computer  will  show  and angle of  rotation  and  an

offset to  the center  of the part that a robot would

have  to  move  to  place  the part  in  the  desired

location.  This  program assumes you would  want  the

part in the exact center of the pad.

Notes:   The   part  must cover the pad from one side

to  the  other.  Also the part must be placed on  the

pad from row zero up for  this  algorithm  to   work.

Only  one direction and the angle  was  given because

the  other  direction  would  be  a  part   dependent

calculation.

5) Help.

Shows this information.

6) End program.

Stops execution of the program.

General notes:

The   sensor   must be on when the program is  executed  and

must   remain  on until  you  are done with the program (you

cannot   turn  the  sensor  on   and  off   during   program

operation).

You  can use the print screen key to get a printout of the 3

dimensional view.

-2-

Basic Program Notes

Line #         Comments

1000           On any error call the error trapping subroutine

1020-1160      Print opening screen

1190           Dimension all Arrays

1200-1220      read in color scale data (at line 3090)

1230           Open communication port (Com1:) for use  at  9600

baud, no parity, 8 data bits, 2 stop bits.

1240           Command sensor to send an initial scan (all  data

from sensor should be 0)

1250-1290      Read in data from sensor and discard it

1320-1360      Inform user that initialization is done and ask if

they want more information on the program

1390-1460      Print out command menu

1470-1490      Get command choice and make sure it is valid

1500           go to subroutines for different commands

1510           reprint the menu

1550-1640      print  border  and color scale  for  sensor  data

displays

1650           Open disk file if command #1 was chosen from menu.

The new data will be saved over any existing file

named "Lord.Dat".

1680           Command to take a scan of the sensor pad.

1690-1800      Gets information from the sensor and changes the

information into number that are stored in a two

dimensional array. If command 1 was the last

chosen the data will also be saved to disk.

-3-

1810           Close data file if opened

1820-1840      Wait until user is done looking at data.

1850           return to origin

1880           run the read data from disk subroutine

1900-1910      go into hi-res graphics and clear the screen

1930-2040      plot x,y,z co-ordinates and record them as 2

dimensional data

2070-2120      clear screen and replot data as a x-direction

grid.

2150-2210      overlay a y-direction grid over the x-grid

2220-2240      wait for user to hit any key and return to  line

1510

2270-2350      Read data file (created by command #1) and return

2380           Clear variables area(1) and area(2)

2390           read data from disk

2400-2410      get tolerance and change it to percent

2420-2460      copy array P into array C

2470           goto  subroutine  to  get data  from  sensor  and

display it

2480-2530      calculate area of both the part on disk and the

one just read from the sensor (each pixel that is

on has an area of .071^2)

2540           compare new part to old part with tolerances added

2550-2580      print results of test

2590-2610      wait for user input then return to menu

2640           clear Y array

-4-

2650           goto  subroutine  to get  data  from  sensor  and

display it

2660-2710      find the last pixel in columns 1,5,6 and 10 and

record their location.

2720           check to see if angle of rotation is 90 degrees if

so goto line 2740

2730-2760      calculate the angle of rotation (and convert it to

degrees)

2770           print angle of rotation

2780-2790      calculate the distance from center of the sensor

the part

2800           Print the distance

2810-2830      waits for user input then return

2860-3020      read help file from disk and display it

3050-3080      Error trapping (only traps i/o errors at lines

1250 and 1270)

3090           data for color scale

Comments

The  program can be modified so that the results of the edge

detection and the parts comparison can be sent to a robot.  To do

this  you  must have two communications ports  on  the  IBM.  Any

information you want the robot to receive could be sent by way of

Com2:.

The  program  was  written in GWBASIC version 3.2  and  then

compiled in Microsoft Quick Basic Compiler 1.02.

-5-

Program Listing

Appendix A

Autoexec.bat File

Appendix B

Power Supply and Cable

Appendix C